Practical Concept Note for GSMA/UCC

Title: On-Device GSM Signal Denoising for Rural Uganda: Extending 2G Coverage Without New Towers

Problem Statement (Add this to problem statement)

Over 11M Ugandans in rural areas rely on 2G GSM for Mobile Money, health alerts, and agricultural info. UCC 2024 data shows 30-40% USSD/SMS failure rates in districts >8km from towers due to low SNR. Building new towers costs $150k-$250k each and requires grid power. Communities need a solution that works with existing handsets and infrastructure today.

Our Innovation (beyond the models)

We are developing lightweight AI models for GSM signal denoising and modulation classification, optimized to run entirely on low-cost Android phones with a $25 software-defined radio dongle. The system is expected to process raw RF in real time, achieving 3-5dB SNR gain on weak GSM signals. This is equivalent to a 30% increase in usable cell radius with zero changes to operator infrastructure.

Evidence to Date

Lab evaluation on captured Ugandan GSM bands shows: 4.2dB average SNR improvement, burst decode rate increase from 52% to 84% at -102dBm. Models will be  quantized to <1.2MB total and run at <40ms latency on Tecno Spark phones common in rural Uganda. Field pre-test to be carried out in Wakiso District to show call setup success improvements expected from 9/20 to 16/20 attempts at a known weak-signal site.

Proposed Pilot: 6-8 Months, 3 Districts

Deploy: 20 kits (Android phone + SDR + antenna = $95/unit) to Village Health Teams and farmer cooperatives in Iganga, Kiryandongo, and Kabale.

Measure: USSD completion rate, SMS delivery time, user-reported “could communicate” vs baseline.

Target: 25% reduction in failed sessions.

Output: Open-source Android APK, training manual in English/Luganda, technical report for UCC on coverage extension via software.

Scalability & Sustainability

Software-only after initial kit.

APK can be side-loaded to any OTG-capable Android.

Local tech shops can assemble kits from parts available on William St, Kampala.

Training should take 2 hours.

Total cost to equip 1000 VHTs <$100k vs $200k for one new tower.

Aligns with UCC Universal Access goals and GSMA’s “Innovation for Rural Connectivity” focus.

Request

We request $18,000 USD (+ the rest of the other budget) to fund pilot kits, transport, field allowances, and 1 research engineer for 6 months.

Co-funding: Kyambogo University provides lab, staff time, and model development in-kind.

Team

Team with 7+ years in wireless DSP + ML.

Partners: Kyambogo University DEEE/Makerere University COCIS, [Rural Health NGO TBD]. Team has prior experience deploying SIM800/ESP32 systems for agriculture in Eastern Uganda.

Conclusion: This is a software-defined coverage solution. It turns the phones already in people’s pockets into smarter receivers, delivering immediate impact while 4G/5G roll-out continues. We can demo live in Kampala with 2 weeks’ notice.